PROJECT REPORT

FAVOUR OKWUDILI      23040102006

AISHAT FADEKEMI GBEMISOLA ONAKOYA     22040102216

OMAR ASLAN   22040102144

TAREQ GALAL   22040102184

OSAMA MAHMOUND DERBALA   22040102139

GitHub Link: https://github.com/missfavour97

ASYNCHRONOUS BIDIRECTIONAL HANDSHAKING SYSTEM USING POLLING

INTRODUCTION:

This project presents a novel asynchronous data exchange system implemented using the DEEDS Digital Circuit Simulator. The primary objective is to implement a reliable data transfer protocol using Asynchronous Bidirectional Handshaking and Polling techniques. This ensures that data is only transferred when both the sender (Manager) and receiver (Worker) are synchronized, preventing data loss or duplication.The project extends and modifies the textbook examples by integrating multiple techniques and introducing role-based control. All functionality is implemented using DMC8 assembly language.

## SYSTEM ARCHITECTURE: The hardware configuration consists of two distinct DMC8 microcomputers connected via a shared data bus and dedicated control lines.

### HARDWARE COMPONENTS: The system consists of;

- Two independent DMC8 CPUs

- A shared 8-bit parallel data bus

- Input switches connected to the Manager

- LED output connected to the Worker

- Two control signal:

  - STB(Strobe): Manager → Worker

  - ACK(Acknowledge): Worker → Manager

## HANDSHAKING LOGIC AND SOFTWARE IMPLEMENTATION

The project employs a "Four-Phase" handshake cycle implemented through DMC8 Assembly code.

### Manager Operation (Polling Mode): The Manager code is designed to be proactive. It first reads the physical switches at Port IA and places that value on the bus via Port OA. It then enters the Polling Phase, where it continuously checks the ACK line (Bit 1 of Port IB) to ensure the Worker is ready.

- Strobe Phase: Once the Worker is ready, the Manager sets STB high.

- Wait Phase: It waits for the Worker to pull ACK high before resetting the STB line and starting a new cycle.

### Worker Operation (Listener Mode): The Worker remains in a LISTEN state, polling its Port IB (Bit 0) for the STB signal.

- Data Capture: Upon detecting STB, it reads the data from the bus and outputs it to the LEDs.

- Acknowledge: It sets ACK high to inform the Manager the task is done.

- Release: It waits for the Manager to drop STB before it drops its own ACK signal, completing the bidirectional loop.

## Conclusion:

This project successfully demonstrates the integration of hardware and software to solve synchronization issues in digital systems. The Asynchronous Bidirectional Handshaking protocol ensures that the Manager and Worker can communicate reliably regardless of their individual execution speeds. The DEEDS simulation confirms that hardware-level handshake logic, paired with efficient assembly polling, creates a stable and scalable architecture for asynchronous data exchange.